# Development and Validation of MATLAB-Based 3D Multibody Kinematics and Dynamics Simulation Engine

Michael F. Vignos

University of Wisconsin-Madison

Developed for Mechanical Engineering 751: Advanced Computational Multibody Dynamics

Table of Contents

1.	Introduction	2

2.	Problem Statement	2

3.	simEngine3D Framework	2

3.1 Organization of GitHub Repository	3

3.2 Object-Oriented Programming	3

multibodySystem.m	3

body.m	4

Basic Constraint Classes: CDconstraint.m, Dconstraint.m, DP1constraint.m, and DP2constraint.m	4

simEngine3DUtilities.m	4

plot Folder	4

3.3 Example Model Definition	4

4.	System Constraints	5

4.1. Kinematic Constraints	5

4.1.1. Basic Constraints	6

4.1.2. Intermediate Constraints	7

4.1.3. Joints	7

4.2. Driving Constraints	9

5.	Externally Applied Forces and Torques	9

5.1. Constant Forces and Torques	9

5.2. Variable Torques	10

5.3. Translational-Spring-Damper-Actuators	11

6.	Analysis of Mechanisms	12

6.1. Prescribing Initial Positions and Orientations	12

6.2. Assembly Analysis	13

6.3. Prescribing Initial Velocities	14

Case 1: Initial velocity known for all bodies.	14

Case 2: Initial velocity not known for any of the bodies.	14

Case 3: Initial velocity known for a subset of the bodies.	14

6.4. Kinematics Analysis	15

6.5. Inverse Dynamics Analysis	16

6.6. Dynamics Analysis	16

7.	Validation Efforts	16

7.1. Validation of Model Components	17

7.1.1. Joints	17

4.1.2. Force and Torque Component Validation	36

Comparison to Benchmark Problems	41

# Introduction

Include a sentence or two about the purpose of this document

State that the purpose of this document is not to  go into a very detailed, technical description of theory used to develop this simulation engine, but rather to give the reader a general understanding of how to use this engine, the capabilities and features of this engine, and an overview of the efforts to validate this simulation engine.

# Problem Statement

The goal of this work was to develop a 3D multibody kinematics and dynamics simulation engine within MATLAB. The theory used to develop this simulation engine was taken from the following sources:

- Haug, Edward J. Computer Aided Kinematics and Dynamics of Mechanical Systems. Boston: Allyn and Bacon, 1989. Print.

- Negrut, Dan. Mechanical Engineering 751: Advanced Computational Multibody Dynamics. University of Wisconsin-Madison, Madison, WI. Fall 2016. Lecture.

# simEngine3D Framework

As previously mentioned, all code for this simulation engine was developed using MATLAB R2014b. It is likely that simEngine3D will perform properly for future versions of MATLAB as the functions used are not version specific. However, this has not been verified.

## 3.1 Organization of GitHub Repository

The GitHub repository for this code is named simEngine3D-Vignos. Within this repository there are 3 main folders: simEngine3DCode, testExamples, and ME751assignments. The folder simEngine3DCode contains all of the MATLAB code that is used to run this simulation engine. The folder testExamples contains example driver files that were used to validate this simulation engine. These files contain examples of kinematics, dynamics, and inverse dynamics analyses. The results of these driver files were compared to results of either other validated simulation packages, the results reported in Computer Aided Kinematics and Dynamics of Mechanical Systems (1), or analytical results computed by hand for validation.

## 3.2 Object-Oriented Programming

The framework of simEngine3D was written using object-oriented programming. Using object-oriented programming allows for the creation of MATLAB classes, which can be leveraged by the user to develop a multibody system in a hierarchical manner (Fig. 1). The classes that exist in this code are contained within the folder simEngine3DCode and are as follows: multibodySystem.m, body.m, CDconstraint.m, Dconstraint.m, DP1constraint.m, DP2constraint.m, and simEngine3DUtilities.m. In addition to these classes, there is also a plot folder that contains various functions that can be used to display and animated the multibody system.  The purpose of each class and the plot commands will be covered in a bit more detail in the following sections.

Figure 1: Outline of hierarchical structure created through the use of classes in developing simEngine3D. The highest level class is the multibody system. A multibody system is made of a set of bodies, which are constrained to each other using joints. Each joint is then comprised of a set of basic constraints. The basic constraints vary depending on which joint is used.

### 3.2.1. multibodySystem Class

An instance of this class contains all of the bodies, constraints, and externally applied forces and torques that are used to perform a simulation. This class also contains all of the methods needed to perform kinematics, inverse dynamics, and dynamics analyses. Following completion of a simulation, the instance of this class used to define the multibody system will contain all of the data stored throughout the simulation (e.g. kinematics of each body, reaction forces and torques, etc.). Additionally, since the these data are stored at each time step, a simulation can be stopped prematurely and the data contained within the instance of this class can be visualized to see what was occurring with the simulation up to the stopping time.

### 3.2.2. body Class

An instance of this class contains all of the attributes of a body (e.g. mass, body number, kinematics, etc.) that are used when performing a simulation. This class also contains functions that are commonly used to compute variables related to a single body (e.g. the orientation matrix of a body, the total torque applied to a body, etc.). Additionally, the state information throughout a simulation is stored within a body class. This approach of having each body in the system defined as its own class allows for improved organization and makes it easier to compute state information for a single body in post-processing.

### 3.2.3. Basic Constraint Classes: CDconstraint, Dconstraint, DP1constraint, and DP2constraint

Instances of each of these classes are similar in that they define the bodies impacted by each constraint, they contain the attributes of each of the four basic constraints, and they contain functions used to compute attributes of these constraints that are needed when performing a simulation (e.g. current state of the constraint, the right hand side of the acceleration equation, partial derivatives of the constraint, etc.). All higher level constraints (i.e. joints) used in defining a multibody system are composed of instances of these basic constraints.

### 3.2.4. simEngine3DUtilities.m

This class is simply a collection of functions that are commonly used when performing a simulation (e.g. computing the distance between two points, computing a skew symmetric matrix from a vector, etc.).

### 3.2.5 plot Folder

This folder contains a collection of functions that can be used to display the position of all bodies in a system at a specific state or to create an animation to visualize the output of a simulation.

## 3.3 Example Model Definition

Figure 2 below contains a screen shot of an example model definition within a driver script. This model is a simple pendulum with two bodies (the mass and the ground) and a revolute joint defined between them. The revolute joint is actually composed of 5 basic constraints. This concept of defining joints using basic constraints will be further discussed in the following section. More thorough examples of model definitions can be found in the testExamples folder in the simEngine3D-Vignos repository.

Figure 2: Example driver file showing the definition of a simple pendulum. This example contains two bodies (the ground and a point mass) and a revolute joint between them.

# System Constraints

## 4.1. Kinematic Constraints

Kinematic constraints are constraints that are added into a multibody system to define the geometry of the motion of the system. In other words, kinematic constraints between two bodies define the relative motion that is allowed between these bodies (i.e. they remove degrees of freedom of each body). Within simEngine3D there are three different levels of constraints: basic constraints, intermediate constraints, and joints. Both intermediate constraints and joints are made up of a collection of basic constraints. When developing a model, it is most common to define constraints between bodies using joints, as the definition of these joints are most easily understood.

### 4.1.1. Basic Constraints

As previously discussed, there are four basic constraints that are used as the foundation to all kinematic constraints in simEngine3D. These constraints are a coordinate distance constraint (CD constraint), a distance constraint (D constraint), a dot product one constraint (DP1 constraint), and a dot product two constraint (DP2 constraint). Each of these basic constraints removes one degree of freedom from a multibody system when it is added into the system.

#### Coordinate Difference (CD) Constraint

A CD constraint defines the distance between two bodies for a single Cartesian coordinate (x, y, or z). The attributes for this constraint are as follows:

- The two bodies to which the constraint is applied (i.e. body 1 and body 2)

- A point attached to body 1 in the body 1 reference frame

- A point v body 2 in the body 2 reference frame

- The Cartesian coordinate that is being constrained

- The prescribed distance between the coordinates of the two bodies

#### Distance (D) Constraint

A D constraint defines the distance between two bodies. The attributes for this constraint are as follows:

- The two bodies to which the constraint is applied (i.e. body 1 and body 2)

- A point attached to body 1 in the body 1 reference frame

- A point attached to body 2 in the body 2 reference frame

- The prescribed squared distance between the two bodies. It is important to note that in the implementation of this constraint the user must provide the square of the distance.

#### Dot Product One (DP1) Constraint

A DP1 constraint constrains the dot product between two vectors, one attached to body 1 and one attached to body 2, to be equal to a prescribed function. This function is commonly set to zero, which constrains the two vectors to be orthogonal. The attributes for this constraint are as follows:

- The two bodies to which the constraint is applied (i.e. body 1 and body 2)

- A vector attached to body 1 in the body 1 reference frame

- A vector attached to body 2 in the body 2 reference frame

- A function prescribing the value of the dot product between the two vectors. If these vectors are defined to be orthogonal, this function must be zero.

#### Dot Product Two (DP2) Constraint

Similar to a DP1 constraint, a DP2 constraint constrains the dot product between two vectors to be equal to a prescribed function. However, in this case one vector is attached to body 1 and the second vector is defined as the vector between two points. One of these points is attached to body 1 and the other is attached to body 2.  The attributes for this constraint are as follows:

- The two bodies to which the constraint is applied (i.e. body 1 and body 2)

- A vector attached to body 1 in the body 1 reference frame

- A point attached to body 1 in the body 1 reference frame

- A point attached to body 2 in the body 2 reference frame

- A function prescribing the value of the dot product between the two vectors. If these vectors are defined to be orthogonal, this function must be zero.

### 4.1.2. Intermediate Constraints

In simEngine3D there are two intermediate constraints: perpendicular one (B1) constraint and perpendicular two (B2) constraint. Both of these constraints are composed of two basic constraints.

#### Perpendicular One (B1) Constraint

A B1 constraint constrains a vector attached to body 2 to be orthogonal to a plane attached to body 1. The plane on body 1 is defined by two vectors attached to body 1. The attributes for this constraint are as follows:

- The two bodies to which the constraint is applied (i.e. body 1 and body 2)

- A vector attached to body 1 in the body 1 reference frame

- Another vector attached to body 1 in the body 1 reference frame. This vector and the previous vector define the plane on body 1.

- A vector attached to body 2 in the body 2 reference frame

#### Perpendicular Two (B2) Constraint

Similar to a B1 constraint, a B2 constraint constrains a vector to be orthogonal to a plane attached to body 1. However, in this case the vector is defined as the vector between a point attached to body 1 and a point attached to body 2. The attributes for this constraint are as follows:

- The two bodies to which the constraint is applied (i.e. body 1 and body 2)

- A vector attached to body 1 in the body 1 reference frame

- Another vector attached to body 1 in the body 1 reference frame. This vector and the previous vector define the plane on body 1.

- A point attached to body 1 in the body 1 reference frame

- A point attached to body 2 in the body 2 reference frame

### 4.1.3. Joints

To date, there are six joints that have been implemented and validated in simEngine3D. Each of these joints are constructed from a set of basic constraints. Below is a description of the attributes needed to define each of these joints. However, to gain a better understanding of how the basic constraints are used to create each of these joints, please see the methods in the multibodySystem class used to create each of these joints (e.g. createSphericalJoint(), createCylindricalJoint(), etc.)

#### Spherical Joint Attributes

- The two bodies connected by this joint (i.e. body 1 and body 2)

- A point attached to body 1 in the body 1 reference frame.

- A point attached to body 2 in the body 2 reference frame. This second point will be constrained to be in the same location as the point defined in 2.

#### Revolute Joint Attributes

- The two bodies connected by this joint (i.e. body 1 and body 2)

- A point attached to body 1 in the body 1 reference frame.

- A point attached to body 2 in the body 2 reference frame. This second point will be constrained to be in the same location as the point defined in 2.

- A vector attached to body 1 in the body 1 reference frame. This vector is the 1st vector used to define the plane in which the revolute joint moves.

- A second vector attached to body 1 in the body 1 reference frame. This vector is the 2nd vector used to define the plane in which the revolute joint moves.

- A vector attached to body 2 in the body 2 reference frame. This vector is orthogonal to the plane on body 1 (i.e. the plane defined by the vectors defined in 4 and 5). In other words, this is the vector about which the revolute joint rotates.

#### Cylindrical Joint Attributes

- The two bodies connected by this joint (i.e. body 1 and body 2)

- A point attached to body 1 in the body 1 reference frame that is along the translational axis of body 1.

- A point attached to body 2 in the body 2 reference frame that is along the translational axis of body 2.

- A vector attached to body 1 in the body 1 reference frame. This vector is the 1st vector used to define the plane orthogonal to the translational axis of this joint.

- A second vector attached to body 1 in the body 1 reference frame. This vector is the 2nd vector used to define the plane orthogonal to the translational axis of this joint.

- A vector attached to body 2 in the body 2 reference frame. This vector is orthogonal to the plane on body 1 (i.e. the plane defined by the vectors defined in 4 and 5).

#### Translational Joint Attributes

- The two bodies connected by this joint (i.e. body 1 and body 2).

- A point attached to body 1 in the body 1 reference frame that is along the translational axis of body 1.

- A point attached to body 2 in the body 2 reference frame that is along the translational axis of body 2.

- A vector attached to body 1 in the body 1 reference frame. This vector is the 1st vector used to define the plane orthogonal to the translational axis of this joint.

- A second vector attached to body 1 in the body 1 reference frame. This vector is the 2nd vector used to define the plane orthogonal to the translational axis of this joint.

- A vector attached to body 2 in the body 2 reference frame. This vector is orthogonal to the plane on body 1 (i.e. the plane defined by the vectors defined in 4 and 5).

- A second vector attached to body 2 in the body 2 reference frame. This vector is parallel to the plane on body 1 (i.e. the plane defined by the vectors defined in 4 and 5) and perpendicular to the vector defined in 4.

#### Universal Joint Attributes

- The two bodies connected by this joint (i.e. body 1 and body 2)

- A point attached to body 1 in the body 1 reference frame

- A point attached to body 2 in the body 2 reference frame. This second point will be constrained to be in the same location as the point defined in 2.

- A vector attached to body 1 in the body 1 reference frame that defines the rotation axis of this joint for body 1.

- A vector attached to body 2 in the body 2 reference frame that defines the rotation axis of this joint for body 2. This vector is orthogonal to the vector defined in 4.

#### Revolute-Cylindrical Joint Attributes

Note: When defining a revolute-cylindrical joint, the two origins of the bodies that are connected by this joint typically also need to be constrained to one another using a distance constraint.

- The two bodies connected by this joint (i.e. body 1 and body 2)

- A point attached to body 1 in the body 1 reference frame that is along the translational axis of body 1.

- A point attached to body 2 in the body 2 reference frame that is along the translational axis of body 2.

- A vector attached to body 1 in the body 1 reference frame that defines the rotational axis of the revolute portion of the joint.

- A vector attached to body 2 in the body 2 reference frame that is along the translational axis of this joint. This vector is orthogonal to the vector defined in 4.

- A vector attached to body 2 in the body 2 reference frame. This vector is the 1st vector used to define the plane orthogonal to the translational axis of this joint. This vector is also orthogonal to the vector defined in 5.

- A second vector attached to body 2 in the body 2 reference frame. This vector is the 2nd vector used to define the plane orthogonal to the translational axis of this joint. This vector is also orthogonal to the vector defined in 5.

## 4.2. Driving Constraints

A driving constraint is used to specify the time-dependent behavior of one of the bodies in the multibody system. To date, all driving constraints in simEngine3D are prescribed by providing a time-dependent function for a basic constraint. As described in the Basic Constraint section, each basic constraint has a function as an input that defines the value of the constraint. By setting this input to be a time-dependent function, a basic constraint can be used as a driving constraint. For example, a DP1 constraint can be used to drive a revolute joint by prescribing the angle with respect to time of the free degree of freedom of this joint.

# Externally Applied Forces and Torques

Within simEngine3D there are three different options for applying external forces and torques to a body. These options consist of applying a constant force or torque, applying a variable torque, or using a translational-spring-damper-actuator.

## 5.1. Constant Forces and Torques

Constant forces are externally applied forces that do not change magnitude or direction, with respect to the global reference frame, throughout an entire simulation (FIGURE). Constant torques are externally applied torques that do not change magnitude or direction, with respect to the body reference frame, throughout a simulation. With this in mind, the direction of constant forces must be specified in the global reference frame and the direction of constant torques must be defined in the body reference frame.

A special case of a constant force is the gravity force acting on each body. The user does not need to add the gravity force to each body in a multibody system. The force of gravity is added to each body when the body is added to the system (Figure 1). However, the user must properly specify the body mass and the gravity direction to ensure proper magnitude and direction of the gravity force.

Figure 3: Portion of a driver file showing how a constant force (top) and a constant torque (bottom) are added into a multibody system. The variable sys is an instance of the multibody system class.

## 5.2. Variable Torques

A variable torque is a torque that changes magnitude, but maintains a constant direction, with respect to the body reference frame, through a simulation. The behavior of this torque is defined by a user supplied MATLAB function that defines the magnitude of the torque as a function of the multibody system state and/or time (FIGRURE). It is important to note, that although the torque magnitude does not need to depend on both the system state and time, both the instance of the multibodySystem class and the current time of the system must be provided as inputs to this function (FIGURE).

Figure 4: Portion of a driver file showing how to add a variable torque into a multibody system.

Figure 5: Example of a function used to define the output of a variable torque component. As seen in this function, the torque depends on the system state, specifically the y-location of body 5, but does not depend on time. However, both the multibody system and the current time must be provided as inputs to the function.

## 5.3. Translational-Spring-Damper-Actuators

A translational-spring-damper-actuator (TSDA) is attached between two points on two different bodies and applies a force based on the distance between these points, the rate of change of this distance, and the current time in the system (Eq ).

Vector between connection points of TSDA:    dij= rj+AjsjQ-ri-AisiP

Length of TSDA:    l= dijTdij

Rate of change of TSDA length:    l= dijlTrj+AjsjQ-ri-AisiP

Force magnitude:    f=kl- l0+cl+hl,l,t, where

l0=resting length of spring

k=spring stiffness

c=damping coefficient

hl,l,t=actuator function

Force direction:    eij=dijl

Total Force:   F=f*eij

When adding a TSDA to the multibody system, the user must define all attributes of a TSDA even if they intend to only use a single attribute of this force component. For example, a user can add a spring into the system using a TSDA by setting the damping coefficient and the actuator function equal to zero (FIGURE).

Figure 6: Portion of a driver file showing how to add a TSDA force element into a multibody system.

# Analysis of Mechanisms

In simEngine3D there are currently three options for performing an analysis of a mechanism: kinematics, inverse dynamics, and dynamics analyses. Prior to performing any of these analyses, the user must provide initial conditions for the mechanism and perform an assembly analysis to ensure the initial conditions are consistent with the user-specified constraints. Each of these steps is discussed in further detail in the following sections.

## 6.1. Prescribing Initial Positions and Orientations

Prior to performing analysis of the multibody system, the user must specify the initial position and orientation of each body in the global reference frame. In simEngine3D the orientation of a body is defined using Euler parameters (also called quaternions). As it is sometimes difficult to provide the initial Euler parameters of a body, simEngine3D contains a function (A2p()) that converts an orientation matrix to Euler parameters. This allows the user to compute the initial orientation matrix of a body and then convert this orientation matrix into the initial Euler parameters. The initial pose of the body is then specified by calling the function setInitialPose() through the instance of the multibodySystem class (FIGURE SHOWING USE OF SETINITIALPOSE).

Figure 7: Portion of a driver file showing how to set the initial pose of a body using the function setInitialPose().

## 6.2. Assembly Analysis

When setting the initial pose of the system, the user also has the option to perform an assembly analysis. An assembly analysis is used to check if the initial pose is consistent with the constraints that have been imposed on the system. If the initial pose is not consistent, the pose is adjusted to ensure consistency. This is done by solving an optimization problem that minimizes an objective function (EQUATION SHOWING OPTIMIZATION PROBLEM) that aims to find a consistent initial pose that is as close as possible to the user-specified initial pose.

minqΨq,t0,r,

given that: Ψq,t0,r=q-q0Tq-q0+rΦTq,t0Φq,t0

and        Ψq=2q-q0T+2rΦTq,t0Φqq,t0, where

q=vector containing position and orientation of all bodies

q0=guess for initial pose

Φq,t0=matrix of all constraints

Φqq,t0=Jacobian of constraint matrix

t0=initial time

r=positive weighting constant

Within simEngine3D, this optimization problem is solved using the MATLAB built-in function fminunc(), which solves an unconstrained minimization of the provided objective function. While not required to run a simulation, performing an assembly analysis is highly recommended. The user can perform an assembly analysis through the use of a flag specified in the function call for setInitialPose().

## 6.3. Prescribing Initial Velocities

Prior to performing a dynamics analysis (but not for a kinematics or inverse dynamics analysis), the user must prescribe the initial velocities of each body in the system. Prescribing the initial velocities of a system can be broken into three cases and simEngine3D has methods to account for each of these cases.

### Case 1: Initial velocity known for all bodies.

In this case, the user can specify the initial velocities by calling the function updateSystemState() and providing the velocities as an input to this function (FIGURE).

Figure 8: Portion of a driver file showing how to the set the initial velocities of all bodies, if all are known.

### Case 2: Initial velocity not known for any of the bodies.

This case is handled in a similar fashion to the previous case in that the velocity of each of the bodies is computed and set using the function computeAndSetIntialVelocities(). However, when calling this function, the user provides an empty matrix for each of the function inputs, which informs the function that none of the initial velocities are known (FIGURE). Initial velocities that satisfy the velocity-level constraints are then computed for all bodies in the system (EQUATION).

ΦrΦp0nB x 3nBPrp=-Φt0nB x 1, where

Φr=partial derivative of the constraint matrix with respect to the body positions

Φp=partial derivative of the constraint matrix with respect to the body orientations

P=p1T01 x 4⋯01 x 401 x 4p2T⋯01 x 4⋯⋯⋯⋯01 x 401 x 401 x 4pnBT

rp= vector of initial velocities

Φt=time derivative of constraint matrix

nB=number of bodies in system (not including the ground)

Figure 9: Portion of a driver file showing how to compute and set the initial velocities of all bodies if all are unknown.

### Case 3: Initial velocity known for a subset of the bodies.

In this case, the user can compute and set the initial velocity for the rest of the bodies in the system using the function computeAndSetIntialVelocities() and providing the body numbers with known velocities and the initial velocities of these bodies as inputs to this function (FIGURE). This function then computes the initial velocity for the rest of the bodies, such that these velocities satisfy the velocity-level constraints (EQUATION).

Figure 10: Portion of a driver file showing how to compute and set the initial velocites of a system if some of the initial velocities are known.

## 6.4. Kinematics Analysis

A kinematics analysis is used to compute the position, velocity, and acceleration of each body in a multibody system throughout a simulation. Prior to beginning a kinematics analysis, the user must begin with a multibody system that has zero degrees of freedom (i.e. a fully constrained system). This means that the number of user-specified constraints must be equal to six times the number of bodies in the system (not including the ground). As previously mentioned, simEngine3D uses Euler parameters (or quaternions) to define the body orientation, which actually gives each body seven degrees of freedom. However, simEngine3D automatically includes one Euler parameter normalization constraint per body, which requires the user to constrain only the other six degrees of freedom to perform a kinematics analysis.

A kinematics analysis is composed of three steps: position analysis, velocity analysis, and acceleration analysis. For the first time step of a kinematics analysis, simEngine3D only performs a velocity and an acceleration analysis because the initial position is specified from either the assembly analysis or the user-specified initial pose. In subsequent time steps, all three steps are performed. During the position analysis, simEngine3D solves a nonlinear system of equations to find the current position that satisfies the position-level constraints (EQUATION). This nonlinear system of equations is solved using the Newton-Raphson method. During the velocity and acceleration analyses, simEngine3D solves a linear system of equations to find the current set of velocities and accelerations that satisfy the velocity and acceleration-level constraints, respectively (EQUATIONS). For a more in-depth discussion of how to compute each of the terms needed to solve these systems of equations, please see chapters 3 and 10 in Computer Aided Kinematics and Dynamics of Mechanical Systems.

Position Analysis:  Φq,t= ΦKqΦDq,t=0,

where ΦKq=kinematic constraints

ΦDq,t=dynamic constraints

Velocity Analysis: Φqq=-Φt

Acceleration Analysis: Φqq=-Φqqqq-2Φqtq-Φtt

## 6.5. Inverse Dynamics Analysis

An inverse dynamics analysis is used to compute the kinematics of each body in a multibody system and the reaction forces due to the constraints throughout a simulation. As with a kinematics analysis, prior to performing an inverse dynamics analysis the user must begin with a fully constrained multibody system. An inverse dynamics analysis also proceeds in a similar fashion to a kinematics analysis in that this analysis performs a position analysis, a velocity analysis, and an acceleration analysis. However, following the acceleration analysis, the inverse dynamics analysis performs an additional step and computes the reaction forces and torques for each of the constraints. This allows the user to determine the forces and torques applied to the center of mass of each body due to each of the constraints.

## 6.6. Dynamics Analysis

A dynamics analysis is used to simultaneously compute the kinematics of each body and the reaction forces due to the constraints in a multibody system throughout a simulation. However, in contrast to a kinematics and an inverse dynamics analysis, a dynamics analysis can be performed when the multibody system has free degrees of freedom. This allows the user to study the behavior of a given mechanism when subjected to externally applied forces and torques.

Performing a dynamics analysis requires simEngine3D to solve the Newton-Euler form of the equations of motion at each time step of the simulation (EQUATION).  , which in turn requires the use of a numerical integration method. Within simEngine3D, an implicit numerical integration method (specifically the Backward Differentiation Formula (BDF) method) is used to compute the positions, velocities, and accelerations of the system at each time step. To date, order 1 and order 2 BDF methods have been implemented into simEngine3D and the desired order of the BDF method can be set when performing the dynamics analysis.

In addition to requiring use of a numerical integration method to compute the time-evolution of a system, solving the equations of motion across a discretized time grid also requires simEngine3D to solve a nonlinear system of equations at each time step. To solve this system of equations, simEngine3D offers three different variations of the Newton-Raphson method: full Newton-Raphson, modified Newton-Raphson, and quasi Newton-Raphson. These methods differ in the form of their Jacobian and in the number of times that the Jacobian is computed. Full Newton-Raphson uses the full Jacobian and re-computes the Jacobian at each iteration. Modified Newton-Raphson uses the full Jacobian, but only computes the Jacobian at the first iteration. Quasi Newton-Raphson uses a simplified Jacobian and only computes the Jacobian at the first iteration. Similar to choosing the order of the BDF method, the desired variation of the Newton-Raphson method can be specified when performing the dynamics analysis.

# Validation Efforts

Validation of simEngine3D has been performed primarily through comparison of simulation results to examples from Computer Aided Kinematics and Dynamics of Mechanical Systems and through comparison to benchmark problems (taken from: http://lim.ii.udc.es/mbsbenchmark/). These efforts have allowed for validation of the joints and the external force and torque components currently implemented in simEngine3D. The following is a discussion of each of the validation problems, including the purpose of each problem, the results of the simulation, and comparison to previously validated results.

## 7.1. Validation of Model Components

### 7.1.1. Joints

#### Simple Pendulum with Revolute Joint

Source of example:

http://lim.ii.udc.es/mbsbenchmark/dist/A01/A01_specification.xml.

Purpose of validation:

This problem served to validate the revolute joint implemented in simEngine3D. Additionally, this problem also allowed for comparison to a common benchmark problem.

Brief overview of validation problem:

The problem is a simple pendulum that consists of a point mass connected to the ground via a revolute joint.

Image of mechanism:

Simulation Parameters:

Start time =

End time =

Step-size =

BDF order =

Variation of Newton-Raphson =

Comparison of results to previously validated results or to analytical results:

Figure 11: Comparison of simulated simple pendulum results to previously validated results for a simple pendulum.

#### Simple Pendulum with Cylindrical Joint

Source of example:

http://lim.ii.udc.es/mbsbenchmark/dist/A01/A01_specification.xml.

Purpose of validation:

The purpose of this example was to validate the implementation of the cylindrical joint.

Brief overview of validation problem:

This problem consists of a simple pendulum with a point mass connected to the ground  a cyliandrical joint. To validate the rotation of the cylindrical joint, the x and y position of the point mass throughout the simulation was compared to the position for a simple pendulum with a revolute joint. To validate the translation of the cylindrical joint, an external force of 1 N was applied to the pendulum mass in the z-direction and it was confirmed that this 1 N load induced a 1 m/s2 acceleration to the mass. This confirmed that the cylindrical was translating as it should.

Image of mechanism:

Simulation Parameters:

Start time =

End time =

Step-size =

BDF order =

Variation of Newton-Raphson =

Comparison of results to previously validated results or to analytical results:

Figure 12: Comparison of results for a simulated simple pendulum with a cylindrical joint to previously validated results.

Figure 13: Kinematics of the mass of the simple pendulum in the z-direction. This behavior is caused by a 1N point load being applied to the mass in the z-direction.

#### Block on Inclined Translation Joint

Source of example:

No specific source for this example. This was just a simple example with an analytical solution to validate the translational joint.

Purpose of validation:

The purpose of this example was to validate the implementation of the translational joint in simEngine3D.

Brief overview of validation problem:

This problem is a block on an inclined plane of 45 degrees. The inclined plane is modeled as a translational joint that passes through the center of mass of the block. Based on the analytical solution, we expect the acceleration of the block in the y-direction to be -4.905 m/s2 and the acceleration in the x-direction to +4.905 m/s2.

Image of mechanism:

Simulation Parameters:

Start time =

End time =

Step-size =

BDF order =

Variation of Newton-Raphson =

Comparison of results to previously validated results or to analytical results:

Figure 14: Kinematics produced by simulating a block sliding down an inclined plane.

#### Slider-Crank Mechanism

Source of example:

Chapter 12.2 of Computer Aided Kinematics and Dynamics of Mechanical Systems

Purpose of validation:

After validation of the revolute and translational joints, this purpose of simulating this mechanism was to validate the spherical and revolute-cylindrical joints implemented in simEngine3D.

Brief overview of validation problem:

This problem consists of two different simulations of a spatial slider-crank mechanism. The first is a dynamics analysis of the mechanism with a constant angular velocity of 2π rad/sec applied to the crank. For this simulation, the kinematics of the slider and the driving torque required to maintain this constant angular velocity were plotted and compared to the textbook results. The second is a dynamics analysis with an initial angular velocity of 6 rad/sec applied to the crank. For this analysis, the kinematics of the slider were plotted and compared to the textbook results. For both simulations, the length of the connecting rod was set to 0.3 m.

Image of mechanism:

Simulation Parameters:

Dynamics analysis with constant angular velocity applied to crank:

Start time =

End time =

Step-size =

BDF order =

Variation of Newton-Raphson =

Dynamics analysis with initial angular velocity applied to crank:

Start time =

End time =

Step-size =

BDF order =

Variation of Newton-Raphson =

Comparison of results to previously validated results or to analytical results:

Dynamics analysis with constant angular velocity applied to crank:

Figure 15: (Top) Textbook results of the x-position of the slider of a spatial slider-crank mechanism for a connecting rod length (l) of 0.3 m. (Bottom) Simulated results of the same mechanism.

Figure 16: (Top) Textbook results of the velocity of the slider of a spatial slider-crank mechanism for a connecting rod length (l) of 0.3 m. (Bottom) Simulated results of the same mechanism.

Figure 17: (Top) Textbook results of the acceleration of the slider of a spatial slider-crank mechanism for a connecting rod length (l) of 0.3 m. (Bottom) Simulated results of the same mechanism.

Figure 18: (Top) Textbook results for the driving torque required to maintain a constant crank angular velocity of 2π rad/sec for a connecting rod length (l) of 0.3 m. (Bottom) Simulated results of the same mechanism.

Dynamics analysis with initial angular velocity of 6 rad/sec applied to crank:

Figure 19: (Top) Textbook results for the x-position of the slider in a spatial slider-crank mechanism with an initial crank angular velocity of 6 rad/sec. (Bottom) Simulated results of the same mechanism.

Figure 20: (Top) Textbook results for the velocity of the slider in a spatial slider-crank mechanism with an initial crank angular velocity of 6 rad/sec. (Bottom) Simulated results of the same mechanism.

Figure 21: : (Top) Textbook results for the acceleration of the slider in a spatial slider-crank mechanism with an initial crank angular velocity of 6 rad/sec. (Bottom) Simulated results of the same mechanism.

#### Four Bar Mechanism

Source of example:

Chapter 12.3 of Computer Aided Kinematics and Dynamics of Mechanical Systems

Purpose of validation:

Prior to simulating this mechanism, the spherical and revolute joints were validated. Therefore, the purpose of simulating this four-bar mechanism was to validate the universal joint and the ability to provide a constant torque to a body.

Brief overview of validation problem:

This problem consisted of two different simulations of a four-bar mechanism. The first simulation was a dynamics analysis with a constant angular velocity of π applied to the first link (the wheel). For validation, the position of point B on the first link, the position and velocity of the rocker, the required driving torque applied to link 1 to maintain this constant angular velocity, and the reaction force at point A on the first link were compared to the textbook results. The second simulation was a dynamics analysis with a constant counterclockwise torque of 10 Nm applied to the first link. For validation, the position and velocity of the rocker were compared to the textbook results.

Image of mechanism:

Simulation Parameters:

Dynamics analysis with constant angular velocity:

Start time =

End time =

Step-size =

BDF order =

Variation of Newton-Raphson =

Dynamics analysis with constant counterclockwise torque:

Start time =

End time =

Step-size =

BDF order =

Variation of Newton-Raphson =

Comparison of results to previously validated results or to analytical results:

Dynamics analysis with constant angular velocity:

Figure 22: (Top) Textbook results for the position of point B in the four-bar mechanism when a constant angular velocity of π rad/sec is applied to the first link. (Bottom) Simulated results of the same mechanism.

Figure 23: (Top) Textbook results for the position of the rocker in the four-bar mechanism when a constant angular velocity of π rad/sec is applied to the first link. (Bottom) Simulated results of the same mechanism.

Figure 24: (Top) Textbook results of the velocity of the rocker in the four-bar mechanism when a constant angular velocity of π rad/sec is applied to the first link. (Bottom) Simulated results of the same mechanism.

Figure 25: (Top) Textbook results for the driving torque applied at point A of the first link in the four-bar mechanism required to maintain a constant angular velocity of π rad/sec of the first link. (Bottom) Simulated results of the same mechanism.

Figure 26: (Top) Textbook results for the x-component of the reaction force at point A in the four-bar mechanism when a constant angular velocity of π rad/sec is applied to the first link. (Bottom) Simulated results of the same mechanism.

Dynamics analysis with constant counterclockwise torque:

Figure 27: (Top) Textbook results for the z-position of the rocker in the four-bar mechanism when a constant torque 10 Nm is applied to the first link. (Bottom) Simulated results of the same mechanism.

Figure 28: (Top) Textbook results for the z-velocity of the rocker in the four-bar mechanism when a constant torque 10 Nm is applied to the first link. (Bottom) Simulated results of the same mechanism.

### 4.1.2. Force and Torque Component Validation

#### Spring-Mass-Damper

Source of example:

No specific source for this example. This mechanism is a commonly seen mass-spring-damper system.

Purpose of validation:

The purpose of simulating this mechanism was to validate the translational-spring-damper-actuator (TSDA) force element.

Brief overview of validation problem:

This problem is a classic mass-spring-damper system in which a spring-damper element is attached to a mass that translates along a frictionless surface. A translational joint was used to allow translation of the mass along only the global x-axis. The analytical solution of the mass displacement was computed by solving the second-order ordinary differential equation that represents this system using ODE45 in MATLAB.  The parameters of the mass-spring-damper are as follows:

mass = 1 kg; k = 100 N/m; c = 2 kg/s; resting length = 0 m;

Image of mechanism:

Simulation Parameters:

Start time =

End time =

Step-size =

BDF order =

Variation of Newton-Raphson =

Comparison of results to previously validated results or to analytical results:

Figure 29: Comparison of analytical results to simulated results for a mass-spring-damper system.

#### Flyball Governor

Source of example:

Chapter 12.6 of Computer Aided Kinematics and Dynamics of Mechanical Systems

Purpose of validation:

The purpose of simulating this flyball governor mechanism was to validate the ability to provide a variable torque to a body during a simulation.

Brief overview of validation problem:

This problem is a flyball governor mechanism that consists of 4 bodies (not including the ground), a TSDA force element, and two variable torques applied to the shaft (torques not shown in figure). One of the variable torques is a function of time and the other depends on the position of the collar. A definition of these variable torques can be seen in chapter 12.6 of Computer Aided Kinematics and Dynamics of Mechanical Systems or in the functions Te.m and Ts.m within the flyballGovernorFromTextbook folder. The simulation of this mechanism consists of a dynamics analysis with an initial velocity of 11.0174 rad/sec applied to the shaft. For validation, the y-location of the collar and the angular velocity of the shaft were compared to the textbook solutions.

Image of mechanism:

Simulation Parameters:

Start time =

End time =

Step-size =

BDF order =

Variation of Newton-Raphson =

Comparison of results to previously validated results or to analytical results:

Figure 30: (Top) Textbook results for the y-position of the slider in the flyball governor mechanism with three different variable torques (different values of C) applied to the axis. (Bottom) Simulated results of the same mechanism.

Figure 31: (Top) Textbook results for the angular velocity of the axis  in the flyball governor mechanism with three different variable torques (different values of C) applied to the axis. (Bottom) Simulated results of the same mechanism.

## Comparison to Benchmark Problems

#### Simple Pendulum with Revolute Joint

Simulation of this mechanism was already discussed at the beginning of section 7. However, it is also listed here because, in conjunction with validation the revolute joint, this mechanism is also a common benchmark problem.

#### N-Four Bar Mechanism

Source of example:

http://lim.ii.udc.es/mbsbenchmark/dist/A02/A02_specification.xml

Purpose of validation:

The purpose of simulating this mechanism was to compare to a common benchmark problem.

Brief overview of validation problem:

This problem consists of an N-four bar mechanism, where N is the number of four bar mechanisms that are combined to create the overall system. The simulation of this mechanism consisted of a dynamic simulation with an initial velocity of 1 m/s in the +X direction for point B0 (labeled in the below image).

Image of mechanism:

v0 = 1 m/s

Simulation Parameters:

Start time =

End time =

Step-size =

BDF order =

Variation of Newton-Raphson =

Comparison of results to previously validated results or to analytical results:

Figure 32: Previously validated results for the position of point B0 (shown in figure) compared to simulated results for an N-four bar mechanism.

Things to do still:

Implement simple pendulum

Implement N-bar mechanism

Implement method to prescribe initial velocities

Implement example to validate a cylindrical joint (could maybe be a simple pendulum with a cylindrical joint instead of a revolute joint??)

Implement method to prescribe angular velocity at a joint, rather than using a DP1 constraint. A DP1 constraint is plagued with too many singularity issue.

Implement method to remove redundant constraints?? Not sure how robust my method is.

Implement ability to read model parameters from a file???

Flyball governor mechanism?

Bricard’s mechanism?

Andrew’s mechanism?

Things done:

Implemented ability to prescribe all basic constraints, all intermediate constraints, and all joints discussed in class.

Implemented ability to perform kinematics, inverse dynamics, and dynamics analyses.

Validated all joints

Validated ability to prescribe constant torque
Implemented kinematics, inverse dynamics, and dynamics analyses

Implemented different methods of computing iteration matrix

Implemented various test cases validated by book and by group that created the benchmark problems

Implemented method to prescribe initial velocities for dynamics analysis.